- DevSuite Software - Internal Release Notes for 6.1.59

(Version 6.1.59.61383 - build date 07/8/2025)

- Special Notices

- Starting with Release 6.0.37

- Due to a change in one of our support libraries, ALL the software must be uninstalled and installers rerun.

- This only must be performed one time.

    - Installers can be at different revisions, but at a minimum must be at 6.0.37.

- Known Issues

- None.

- Changes for Release 6.1.59 (Full list can be found in next section)

- New Sensor Support

      - None

- New Features

  - Added SERDES setup ini file for Microchip.

  - Fix asymmetry in local FPN calculations.

- General Changes

    - For apbase.Camera().grab_frame(), make splitting virtual channels an option, for backward compatibility.

    - Higher dark level sanity check value.

    - Documentation

  - No changes.

  - INI File Updates

      - AP1302 Rev2; WIP 32585 -  Update from 448 -> 449

      - AR0235CS-REV2; WIP 32577

  - Plug-in

    - STE Plugin; resolved error when trying to load a missing CFG file.

- Tools

  - None.

- FlashTool

- None.

- ModeCalc

- For AR0544CS, Fix missing exp_ratio_direct variable.

- Register Wizard

- AR0521; Fix for MIPI_TIMING_1, minimum value of clk_prepare is 1.

- AR0830; Fix that clk_post was masked as 4 bits when it's really 6 bits.

- AR0830; Sanity checks on MIPI timing values. Fix constraint on VT VCO frequency.

- AR0830; Fix for the Timing check box on the register list.

- AR0830; Hide the non-functional Fixed Frame Rate control (It only applies to certain SOCs.)

- AR0830; mainly make the OP Pixel Divider control work, set CSI_DATA_FORMAT for 8-bit mode, and numerous modifications for frame timing. Not tested.

- AP0102AT-Rev2_AR0136-Rev1.cfg; added STE support.

- SensorTune

  - AR0544; added new RGB-IR QE for AR0544. Measured on CRA11 and CRA33 CSP parts with plain glass.    Glass attenuation factored out.

- Installation Instructions

- Refer to the “DevSuite Quick-Start Guide” for installation and usage instructions. It can be found via the Startup shortcut “All Programs / Aptina Imaging”.

- To Create Bug Reports and Feature Requests:

Click the bug report button  on DevWareX.

Use this link for instructions on how to gather “Crash” information.